Combi – Oven Deliverables

- PCB Board Deliverables

  - Main PCB Board.

  - 6 Pin Cable.

  - Power Measurement PCB Board.

  - 3 CT Coils.

  - 10 Nos 12 mm Bolt nut.

- Flow sensor.

- 1.5 meter 3 core wire

- Temperature Sensor.

- Audio Board.

- Speakers.

- 5 meters of 2 core wires for 2 speakers

- Android Tablet.

12

13

11

10

9

8

7

6

5

4

3

2

1

- Input Power supply

- The 24V SMPS power supply is connected using these two pins.

- An important note is that the SMPS supply should only be connected to the PCB board.

- If any 24V supply is needed, it should be sourced from the PCB board.

- 12 v Output

- This Output is used to power the Audio Module

- VFD DAC Output

- A 10V DAC output is obtained from this pin. The voltage of the DAC output is adjusted based on the fan speed value.

- These two pins are connected to the VFD drive.

- The -Ve is connected to the 0V socket as indicated on the VFD.

- The +Ve is connected to the AI1 socket on the VFD.

- Bluetooth Module

- The HC-05 module facilitates Bluetooth communication between an Android tablet and MCU. The behaviours of the module are outlined below:

- If the red LED blinks continuously, it indicates that Bluetooth is not connected.

- If the LED blinks once every 4 seconds, it signifies that Bluetooth is successfully connected.

- Max6675 Thermocouple Module

- K - Type Thermocouple is used for the Temperature measurement this module is Connected to the Mentioned RMC Socket.

- Flow Sensor

- 45 mm Flow sensor is used for the Flow measurement it should be connected in the first 3 pin RMC socket.

- Charger Output

- The First pin is the relay controlling pin this terminal is connected to the 8th relay in the relay bank

Note: The relay is negative controlled so connect the terminals is negative port of the relay bank.

- Output Controls

This section is solely intended for outputs.

A voltage of 24V will be drawn from the two terminals indicated in the picture.

This voltage will be utilized to control the drain valve and the relay.

- Water Inlet valve

The Water inlet output terminal is connected to the 7th relay in the relay bank

- Heater

The Heater control output is connected to the 6th relay in the relay bank.

- Light

The light control output is connected to the 5th relay in the relay bank.

- Wash pump

The Wash pump control output is connected to the 4th relay in the relay bank.

- Drain valve

The Drain valve control output is connected to the 3rd relay in the relay bank.

- Motor reverse

The Motor Reverse control output is connected to the 2nd relay in the relay bank.

- Motor Forward

The Motor Forward control output is connected to the 1st relay in the relay bank.

- Input section

- First Pin will be used for the proximity sensor input

- Drain valves Grey colour wire is connect to the DV_open Terminal

- Drain Valves Pink Colour wire is connected to the DV_Close Terminal

- VFDs D02 (NO) Socket pin will be connected to the VFD Feedback.

- Connect the Drain valve and Proximity Sensors Ground in the GND terminal.

- Power Measurement Board Connector

- The MCP3008s SPI wire will be connected with this socket. Socket 11 is shown in the Picture is the another point connector.

- 6 pin RMC socket is used Each pins direction is straight to the another one.

- Power Measurement Board Connector

- No 10 socket is connected with this port

- 3 phase Supply connector

- This Board is used to Measure the Voltage and Current Consumption value.

- 3 phases, Phase terminals will be connected to this terminal

- CT coil Terminals

- Three CT coils will be Connected to this Port